User Story A5 – Sensors 2

Documentation of Work Steps

Portable Indoor Feedback System | PIF 2025/2026

# 1. Project Overview

| User Story | A5 – Sensors 2 |
| --- | --- |
| Sprint / KW | KW 09 |
| Project | Portable Indoor Feedback (Wetterstation) |
| Goal | Connect and test the light sensor (BH1750) and environmental sensor (BME280) to obtain data on temperature, air pressure, and brightness. |

# 2. List of Materials

| Component | Description | Quantity |
| --- | --- | --- |
| GY-302 (BH1750) | Light sensor module (I2C) | 1 |
| GY-BME/BMP280 | Barometric sensor module (I2C) – temperature, air pressure, humidity | 1 |
| Socket strip with cable | 5-pin plug-in cable for BH1750 | 1 |
| Socket strip with cable | 6-pin plug-in cable for BME280 | 1 |
| Pin headers | For soldering onto the BME280 module | 1 set |
| Raspberry Pi | Single-board computer (Model 3 B v1.2) | 1 |
| Digilab breadboard / circuit board | For circuit assembly and testing | 1 |
| Plug-in cables (jumper wires) | For connections on the breadboard | several |

# 3. Work Steps

## 3.1 Component Check

Before starting, all required components were checked for completeness and visual integrity:

- GY-302 (BH1750) light sensor module – confirmed present

- GY-BME280 barometric sensor module – confirmed present

- Plug-in cables with socket strips for both sensors – confirmed present

- Pin headers for soldering – confirmed present

## 3.2 Soldering the Light Sensor (BH1750)

The GY-302 light sensor module does not include a pre-soldered cable connection. The socket strip with cable was soldered to the module according to the pin labeling on the PCB:

- Pins on the GY-302: VCC, GND, SCL, SDA (and ADDR)

- The socket strip was aligned and soldered onto the module's through-holes.

- After soldering, the solder joints were inspected for cold solder and short circuits.

## 3.3 Soldering the Barometric Sensor (BME280)

The GY-BME280 module required pin headers to be soldered on so that it could be connected to the plug-in cable:

- Pin headers were inserted into the PCB holes (VCC, GND, SCL, SDA, CSB, SDO).

- Each pin was soldered carefully, ensuring no solder bridges between adjacent pins.

Important note: The pins CSB and SDO are NOT connected in this setup. Only VCC, GND, SCL, and SDA are used.

## 3.4 Circuit Assembly on the Digilab Breadboard

Both sensors were connected to the Raspberry Pi via the Digilab breadboard. The I2C bus lines (SDA and SCL) were distributed in parallel to both sensors:

### BH1750 (GY-302) Wiring

| Module Pin | Raspberry Pi Pin | Physical Pin No. | Wire Color |
| --- | --- | --- | --- |
| VCC | 5V | Pin 2 | Red |
| SDA | GPIO2 | Pin 3 | Green |
| SCL | GPIO3 | Pin 5 | Blue |
| GND | GND | Pin 14 | Black |

### BME280 Wiring

| Module Pin | Raspberry Pi Pin | Physical Pin No. | Wire Color |
| --- | --- | --- | --- |
| VIN | 3.3V | Pin 1 | Red |
| GND | GND | Pin 9 | Black |
| SCL | GPIO3 | Pin 5 | Blue |
| SDA | GPIO2 | Pin 3 | Green |
| CSB | – not connected – | – | – |
| SDO | – not connected – | – | – |

Note: The BH1750 uses 5V (Pin 2), while the BME280 uses 3.3V (Pin 1). GND, SDA, and SCL are shared between both sensors in parallel on the breadboard.

# 4. I2C Address Determination and Configuration

Both sensors communicate via the I2C protocol. After the circuit was assembled and the Raspberry Pi was powered on, the I2C addresses were identified using the following command:

sudo i2cdetect -y 1

The I2C scan returned the following addresses:

| Sensor | I2C Address | Notes |
| --- | --- | --- |
| BH1750 (GY-302) | 0x23 | Default address (ADDR pin = GND/floating) |
| BME280 | 0x76 | Default address (SDO = GND) |

No address conflicts were detected. Both sensors were recognized correctly.

# 5. Function Test

The function tests were carried out using the provided test scripts on the Raspberry Pi:

## 5.1 BME280 Test

python3 BME280/test.py

The script successfully read and output the following sensor data:

- Temperature: e.g. 22.4 °C

- Air Pressure: e.g. 1013.2 hPa

- Humidity: e.g. 45.6 %

## 5.2 BH1750 Test

python3 LH1750/test.py

The script successfully read and output the following sensor data:

- Light intensity: e.g. 245.0 lx

Both test programs ran without errors and delivered plausible sensor values. The function test was thus completed successfully.

# 6. Wiring Diagram

The circuit was assembled as shown in the connection diagram from the resources (KiCad schematic, User Story A5, Sheet 3/4). The key points of the diagram are:

- Raspberry Pi GPIO header at the center

- BME280 barometric sensor connected to 3.3V (Pin 1), GND (Pin 9), SCL (GPIO3/Pin 5), SDA (GPIO2/Pin 3)

- GY-302 light sensor connected to 5V (Pin 2), GND, SCL, SDA in parallel

- CSB and SDO pins of the BME280 are left unconnected (marked with X in the schematic)

A photo or diagram of the actual Digilab breadboard circuit is attached separately or included in the project folder.

# 7. Difficulties and Problems

| Problem | Description | Solution |
| --- | --- | --- |
| Pin assignment warning (BME280) | The supplied BME280 module has a different pin order than the reference diagram in the resources. | The actual pin labels on the PCB (VCC, GND, SCL, SDA, CSB, SDO) were carefully read and matched to the correct Raspberry Pi pins before connecting. |
| Voltage difference | The BH1750 requires 5V while the BME280 requires 3.3V. Connecting the wrong voltage could damage the BME280. | The power connections were double-checked before powering on: BH1750 to Pin 2 (5V), BME280 to Pin 1 (3.3V). |
| Soldering quality | First attempt at soldering the socket strip on the BH1750 resulted in a cold solder joint on the SDA pin. | The joint was re-heated and additional solder was applied. The connection was verified with a multimeter for continuity. |

# 8. Summary

All requirements from User Story A5 were successfully completed:

- Socket strip with cable soldered to the BH1750 light sensor module.

- Pin headers soldered to the BME280 barometric sensor module.

- Circuit assembled on the Digilab breadboard according to the wiring diagram.

- I2C addresses identified (BH1750: 0x23, BME280: 0x76) and confirmed.

- Function test completed successfully with both test scripts.

- Documentation prepared including work steps, materials list, wiring information, and problem descriptions.

The sensors are ready for integration into the Portable Indoor Feedback system and can now deliver reliable data on temperature, air pressure, humidity, and brightness.